Docusign Envelope ID: 3C4A1291-3BAA-811B-8058-4A739AC01D80

DEMONSTRATION DOCUMENT ONLY

PROVIDED BY DOCUSIGN ONLINE SIGNING SERVICE

999 3rd Ave, Suite 1700 • Seattle • Washington 98104 • (206) 219-0200

Cytotoxicity Test Plan www.docusign.com

Doc #: XXX-

XXXXXX-XX

Logo

MEM Elution Method <Add Device Name> Rev.: Page: XX

1 of 2

1. Purpose

The purpose of this study is to evaluate the cytotoxicity of a test article extract using

an in vitro mammalian cell culture test.

2. Testing Guidelines

2.1. Based on the requirements of the International Organization for Standardization

10993-5, Biological evaluation of medical devices – Part 5: Tests for in vitro

cytotoxicity.

2.2. Based on Guidance for Industry and FDA Staff, “Use of International Standard

ISO 10993-1, "Biological evaluation of medical devices - Part 1: Evaluation and

testing within a risk management process", VI. Test-Specific Considerations,

Section

Temperature: 37C

Duration: 24 – 72 hours

Vehicle: allow for extraction of both polar and nonpolar constituents from the

test article

o E.g. mammalian cell culture media (e.g., MEM) supplemented with 5-10% serum

3. GLP Compliance

Good Laboratory Practice – This nonclinical laboratory study will be conducted in

accordance with the United States Food and Drug Administration Good Laboratory

Practice Regulations, 21 CFR Part 58.

4. Identification of Test and Control Articles

4.1. Test Article

evaluated.

The sponsor, <company name> will submit the test articles, <trade name>, to be

4.2. Extraction Vehicle

Single strength Minimum Essential medium supplemented with 5% fetal bovine

serum, 2% antibiotics and 1% L-glutamine (1X MEM)

The extraction conditions shall attempt to exaggerate the clinical use conditions

so as to define the potential toxicological hazard; however, they should not in

any instance cause physical changes such as fusion or melting, which results in

Template Number:

xxx-xxxx-xx Rev A

Printed copies are considered UNCONTROLLED documents after the date printed.

Printed on December 4, 2024

CONFIDENTIAL- PROPRIETARY INFORMATION – Company NameDocusign Envelope ID: 3C4A1291-3BAA-811B-8058-4A739AC01D80

DEMONSTRATION DOCUMENT ONLY

PROVIDED BY DOCUSIGN ONLINE SIGNING SERVICE

999 3rd Ave, Suite 1700 • Seattle • Washington 98104 • (206) 219-0200

Cytotoxicity Test Plan www.docusign.com

Doc #: XXX-

XXXXXX-XX

Logo

MEM Elution Method <Add Device Name> Rev.: Page: XX

2 of 2

a decrease in the available surface area. A slight adherence of the pieces can be

tolerated.

Note: the 1X MEM extraction method is conducted in the presence of serum to

optimize extraction of both polar and non-polar components.

4.3. Control Articles

5. Test system

5.1. Test System and Justification

5.2. Test System Management

6. Method

6.1. Test Article Preparation

6.2. Control Article Preparation

6.3. Extract Observation

6.4. Test Procedure

7. Evaluation and Statistical Analysis

8. Report

The final report will include information on the cell line, culture medium methods, test

and control results, and any additional pertinent observations.

9. Quality Assurance

Inspection will be conducted at intervals adequate to assure the integrity of the study

in conformance with 21 CFR 58.35(b)(3). The final report will also be reviewed for

conformance to Section 58.185, subpart J, of the GLP Regulations.

10. References